Kaylee Auguillard

ITAI 4370

Professor Chiyangwa

17 September 2025

Assignment 02

1. Explain the difference between EPC (4G) and 5GC (5G).

The Evolved Packet Core (EPC) of 4G networks is a monolithic, hardware-centric architecture. In contrast, the 5G Core (5GC) is a modular, software-based core built on a cloud-native, service-based architecture (SBA) (Sammed-Chougule). This shift from dedicated hardware to a software-driven model makes the 5GC more flexible and scalable, allowing it to support diverse use cases that the EPC was not designed for.

2. Why is network slicing important in 5G? Provide real-world examples.

Network slicing is crucial for 5G because it enables the creation of multiple dedicated, virtual networks on a single shared physical infrastructure (Ericsson). This allows mobile operators to customize network performance with specific levels of bandwidth, latency, and security for different applications. For example, a low-latency slice can support autonomous vehicles, while a high-bandwidth slice can handle live video streaming.

3. What is Multi-access Edge Computing (MEC), and how does it reduce latency?

Multi-access Edge Computing (MEC) moves computational resources and services from a centralized cloud to the network’s edge, closer to the end user (HPE). By processing data locally, MEC reduces the physical distance a signal must travel, which significantly lowers latency. This is vital for real-time applications like augmented reality and industrial automation.

4. Describe the role of AI in dynamic resource allocation.

Artificial intelligence (AI) enables dynamic resource allocation by using machine learning to provide real-time optimization within 5G networks (Bikkasani and Yerabolu). Unlike static network configurations, AI continuously analyzes network data to predict traffic and intelligently manage resources. This allows the network to automatically allocate bandwidth and power where they are needed most, ensuring optimal performance and efficiency.

5. In SBA, what is the role of the Network Repository Function (NRF)?

In 5G's Service-Based Architecture (SBA), the Network Repository Function (NRF) serves as a central registry for all other network functions (NFs) (Gooram). Its primary role is to allow NFs to register and discover each other, enabling seamless communication. This functionality is key to the "plug-and-play" nature of 5G networks, allowing new functions to be easily added or updated.

Works Cited

Bikkasani, Dileesh, and Malleswar Yerabolu. “AI-Driven 5G Network Optimization: A Comprehensive Review of Resource Allocation, Traffic Management, and Dynamic Network Slicing.” American Journal of Artificial Intelligence, vol. 8, no. 2, 28 Nov. 2024, pp. 55–62, https://doi.org/10.11648/j.ajai.20240802.14.

Ericsson. “Network Slicing – Give a Slice of 5G.” Www.ericsson.com, 7 June 2021, www.ericsson.com/en/network-slicing.

GooRAM. “5G : What’s the NRF ?” Medium, 8 June 2023, gooram.medium.com/5g-whats-the-nrf-7db7a83cb1bd.

HPE. “What Is Multi Access Edge Computing | Glossary.” Hpe.com, 2025, www.hpe.com/us/en/what-is/multi-access-edge-computing.html.

Sammed-Chougule. “5G Core vs. 4G EPC: A Deep Dive into the Evolution of Mobile Networks.” Medium, 4 May 2025, medium.com/@sammedchougule321/5g-core-vs-4g-epc-a-deep-dive-into-the-evolution-of-mobile-networks-3fae679b93c8.