Kaylee Auguillard

ITAI 4370

Professor Chiyangwa

22 September 2025

Assignment 05

AI/ML Applications in RAN

- Radio Resource Management (RRM):
Example: AI/ML can dynamically optimize spectrum allocation by learning traffic patterns. For instance, a machine learning model can predict periods of high demand in specific cells and proactively adjust resource blocks to balance load, reducing congestion and improving throughput.

- Network Reliability:
Example: In Open RAN, AI/ML can detect early signs of link degradation or interference by analyzing performance metrics in real time. The system can automatically trigger rerouting or adjust power levels to prevent dropped connections, improving network uptime.

SON and Predictive Maintenance

- Self-Optimization:
SON continuously monitors KPIs like signal strength, handover success, and user density. It tunes parameters such as power levels, tilt angles, and handover thresholds to maximize coverage and capacity without human intervention.

- Self-Healing:
When a base station fails or underperforms, SON detects anomalies (e.g., sudden KPI drops) and reconfigures nearby cells—like expanding their coverage or reallocating resources—to maintain service until the issue is resolved.

- Predictive Maintenance and QoS:
Predictive maintenance uses AI models to analyze hardware performance trends (e.g., temperature spikes or error logs). By identifying potential failures before they occur, operators can schedule maintenance proactively, preventing service disruptions and ensuring consistent Quality of Service.

Hands-On Lab Application

- Normalizing User Demand:
In the simulation, normalizing user demand scales varying traffic requests into a comparable range (e.g., 0–1). This ensures fair resource allocation, prevents any single user’s demand from dominating, and stabilizes training for optimization algorithms.

- Real-World Relevance:
In Open RAN, similar normalization occurs in schedulers to balance resource allocation among users with diverse traffic patterns. It reflects how real-world systems standardize inputs to improve fairness and efficiency in multi-vendor environments.

Traditional RAN vs. Open RAN

- Two Key Differences:

  - Architecture: Traditional RAN uses proprietary, integrated hardware and software from a single vendor. Open RAN disaggregates components (e.g., RU, DU, CU) with open interfaces.

  - Flexibility: Traditional RAN limits vendor choice, while Open RAN allows mixing vendors for different components, reducing costs and fostering innovation.

- Vendor Interoperability:
Open RAN’s open interfaces (e.g., O-RAN fronthaul) and standards promote seamless integration of equipment from multiple vendors, avoiding vendor lock-in and enabling best-of-breed solutions.

RAN Intelligent Controller (RIC)

- Near-Real-Time RIC:
Operates on the order of ~10 ms–1 s latency to optimize live network performance.
Example Function: Dynamic interference management—adjusting beamforming or handover decisions in near real time.

- Non-Real-Time RIC:
Works with >1 s latency for policy guidance, training AI models, or network planning.
Example Function: Long-term traffic pattern analysis to generate policies that the Near-RT RIC enforces.

Works Cited

3GPP, et al. “AI/ML for NG-RAN & 5G-Advanced towards 6G.” 3gpp.org, 2025, www.3gpp.org/technologies/ai-ml-ngran.

Faisal. “C-RAN vs Cloud RAN vs VRAN vs O-RAN vs Traditional RAN- Guide!” Telcocloudbridge.com, 17 Apr. 2021, telcocloudbridge.com/blog/c-ran-vs-cloud-ran-vs-vran-vs-o-ran/.

IBM. “Predictive Maintenance.” Ibm.com, 9 May 2023, www.ibm.com/think/topics/predictive-maintenance.